UIC-A

Universal ICSP Socket

User's Manual

V1.0

June 2010

Information contained in this publication regarding device applications and the like is intended through suggestion only and may be superseded by updates. It is your responsibility to ensure that your application meets with your specifications. No representation or warranty is given and no liability is assumed by Cytron Technologies Incorporated with respect to the accuracy or use of such information or infringement of patents or other intellectual property rights arising from such use or otherwise. Use of Cytron Technologies’s products as critical components in life support systems is not authorized except with express written approval by Cytron Technologies. No licenses are conveyed, implicitly or otherwise, under any intellectual property rights.

INDEX

- Introduction/Overview							3

- Packing List									4

- Dimension									5

- Board Layout 									6

- Hardware									8

- Warranty									10

1. INTRODUCTION/OVERVIEW

UIC-A (Universal ICSP Adapter) is designed as a programmer’s output socket adapter which are compatible to Cytron UIC00B programmer, Microchip PICkit programmer and Microchip In Circuit Debugger. Each of them has their own standard connection. To ease user, Cytron has created this UIC-A so that user can use any programmer regardless of the development board. For example, a development board which uses PICkit2’s socket standard, which is 6 way header pins can be programmed with UIC00B by using the UIC-A!

2. PACKING LIST

Please check the parts and components according to the packing list. If there are any parts missing, please contact us at sales@cytron.com.my immediately.

- 1 x UIC-A

3. DIMENSION

4. BOARD LAYOUT

Cytron’s ICSP Pinout

| PIN | DESCRIPTION | PIN | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | NC | 6 | GND |
| 2 | NC | 7 | PGD |
| 3 | MCLR | 8 | VDD |
| 4 | GND | 9 | AUX |
| 5 | PGC | 10 | NC |

*NC is not connected

Microchip ICD’s ICSP Standard Pinout

| PIN | DESCRIPTION | PIN | DESCRIPTION |
| --- | --- | --- | --- |
| 1 | MCLR/Vpp | 4 | PGD |
| 2 | VDD | 5 | PGC |
| 3 | GND | 6 | AUX |

5. HARDWARE

This section will show the setup to use UIC-A with UIC00B, PICkit Programmer or Microchip In Circuit Debugger (ICD).

Figure below shows that the Cytron UIC00B Standard can be converted to other type of standard by using UIC-A. Users may use PICkit Standard or IDC Standard to program their ICSP development board. Example below shows Cytron UIC00 Standard was converted to PICkit Out to load program into Microchip development board.

Figure below shows that the PICkit ICSP Standard can be converted to other type of standard by using UIC-A. Users may use Cytron UIC00B Standard or IDC Standard to program their ICSP development board. Example below shows PICkit ICSP Standard was converted to Cytron UIC00B Standard to load program into Cytron’s development board.

Figure below shows that the ICD ICSP Standard can be converted to other type of standard by using UIC-A. Users may use Cytron UIC00B Standard or PICkit Standard to program their ICSP development board. Example below shows ICD Standard was converted to Cytron UIC00B Standard to load program into Cytron’s development board.

6. WARRANTY

- Product warranty is valid for 12 months.

- Warranty only applies to manufacturing defect.

- Damaged caused by misuse is not covered under warranty

- Warranty does not cover freight cost for both ways.

Prepared by

Cytron Technologies Sdn. Bhd.

No. 16, Jalan Industri Ringan Permatang Tinggi 2,

Kawasan Industri Ringan Permatang Tinggi, 
14100 Simpang Ampat,
Penang, Malaysia.

Tel:	+604 - 504 1878

Fax: 	+604 - 504 0138

URL: 	www.cytron.com.my

Email: support@cytron.com.my

sales@cytron.com.my