Top of Form

# HRB8825 Stepper Motor HAT For Raspberry Pi, Drives Two Stepper Motors, Up To 1/32 Microstepping

SKU: 22311
Part Number: Stepper Motor HAT (B)
Brand: Waveshare

$14.99 INCREASEDECREASEADD TO CART

$14.192+

$13.793+

$13.634+

Related Products:

Share:

0

Bottom of Form

DESCRIPTION

PACKAGE CONTENT

HRB8825 Stepper Motor HAT For Raspberry Pi, Drives Two Stepper Motors, Up To 1/32 Microstepping

Stepper Motor HAT (B)

Alternative Solution Of Stepper Motor HAT,
Drives Two Stepper Motors, Up To 1/32 Microstepping

For 3D Printer, Sculpturing machine, Mechanical arm...

Features At A Glance

- Standard Raspberry Pi 40PIN GPIO extension header, supports Raspberry Pi series boards, Jetson Nano

- Onboard dual HR8825 motor controller IC with built-in microstepping indexer, drives two stepper motors, easy to use

- 6 available microstepping modes, configured with the DIP switches: full-step, half-step, 1/4-step, 1/8-step, 1/16-step, and 1/32-step

- Adjustable motor drive current via potentiometer, maximum 2.5A current output

- Protection features: Overcurrent Protection (OCP), Thermal Shutdown (TSD), VM Undervoltage Lockout (UVLO)

- Integrates 5V regulator, allows providing power to Raspberry Pi

- Onboard multi connector options for stepper motors in different specifications

- Comes with development resources and manual (examples in BCM2835, wiringPi, and python)

Specifications

| MOTOR CONTROLLER | HR8825 |
| --- | --- |
| MOTOR DRIVE VOLTAGE | 8.2V~28V |
| MOTOR DRIVE CURRENT | 2.5A |
| LOGIC VOLTAGE | 3.3V |
| MOUNTING HOLE SIZE | 3.0mm |
| DIMENSION | 65 × 56mm |

Raspberry Pi 40PIN GPIO Compatibility

Designed For Raspberry Pi Boards / Jetson Nano

Onboard 2-Ch HR8825 Motor Driver Chip

Built-In Converter For Easy Control, Can Drive Two Stepper Motors

6 Available Microstepping Modes

Configured Via The DIP Switches:
Full-Step, Half-Step, 1/4-Step, 1/8-Step, 1/16-Step, And 1/32-Step

| MODE2 | MODE1 | MODE0 | MOTOR MICROSTEPPING SETTING |
| --- | --- | --- | --- |
| 0 | 0 | 0 | full-step |
| 0 | 0 | 1 | half-step |
| 0 | 1 | 0 | 1/4-step |
| 0 | 1 | 1 | 1/8-step |
| 1 | 0 | 0 | 1/16-step |
| 1 | 0 | 1 | 1/32-step |
| 1 | 1 | 0 | 1/32-step |
| 1 | 1 | 1 | 1/32-step |

Onboard Different Stepper Motor Interfaces

Easy Access To Stepper Motors In Different Specifications

| PIN | MOTOR INTERFACE DESCRIPTION |
| --- | --- |
| VIN | 8.2~28V Power Input |
| 5V | 5V Power |
| GND | Ground |
| A1 | Output 1 terminal of bipolar stepper motor M1 winding A |
| A2 | Output 2 terminal of bipolar stepper motor M1 winding A |
| B1 | Output 1 erminal of bipolar stepper motor M1 winding B |
| B2 | Output 2 terminal of bipolar stepper motor M1 winding B |
| A3 | Output 1 terminal of bipolar stepper motor M2 winding A |
| A4 | Output 2 terminal of bipolar stepper motor M2 winding A |
| B3 | Output 1 terminal of bipolar stepper motor M2 winding B |
| B4 | Output 2 terminal of bipolar stepper motor M2 winding B |

Application Examples

Easy Access To Stepper Motors In Different Specifications

for reference only, Raspberry Pi and stepper motors in the picture are NOT included.

What's On Board

- Raspberry Pi GPIO interface
Can be connected to various Raspberry Pi HATs

- Motor interface
Three different specifications of the motor interface, easy to use

- MP1482 power chip
External power supply can be converted into 5V voltage to power the Raspberry Pi

- HR8825 motor driver chip
Can drive two stepper motors

- External power supply interface
8.2V ~ 28V DC power input

- Motor power switch
You can choose whether the module supplies power to the Raspberry Pi

- DIP switch
For setting 6 different step resolutions

- Potentiometer
Adjustable motor drive current

Pin Definition

Outline Dimensions